# Mini Research Proposal

## Arduino-Based Temperature Monitoring and Warning System

Module: ELYM 127 — Introduction to Electronics

# 1. Introduction

This proposal outlines a small embedded electronics project: a temperature monitoring and warning system built around an Arduino Uno, an NTC thermistor, two indicator LEDs, and a 16x2 I2C liquid crystal display (LCD). The system continuously measures ambient or object temperature and visually communicates the current status to a user — a steady blue LED for normal conditions, and a red LED plus an on-screen warning message when a defined high-temperature threshold is exceeded.

# 2. Background and Motivation

Temperature monitoring is one of the most common real-world applications of basic analogue and digital electronics, and it draws directly on foundational circuit theory: resistive sensing, voltage division, semiconductor devices, and simple digital control. Building a working prototype gives practical, hands-on grounding to concepts that are normally studied only analytically, such as Kirchhoff's laws, series voltage division, and diode behaviour.

# 3. Problem Statement

Many everyday and industrial devices (fridges, incubators, server rooms, engines) require early, low-cost visual warning when a temperature threshold is crossed, without needing a full data-logging or networked system. The aim of this project is to design and build a simple, low-cost circuit that senses temperature and gives immediate, unambiguous visual feedback of the system's status.

# 4. Objectives

- Design a resistive voltage-divider circuit using an NTC thermistor to convert temperature into a readable analogue voltage.

- Interface the sensor with an Arduino Uno's analogue-to-digital converter (ADC) and convert the raw reading into a temperature value in degrees Celsius.

- Use digital output pins to drive LED indicators representing distinct system states (normal vs. warning).

- Display real-time temperature and status text on an I2C-driven LCD.

- Relate each stage of the circuit and code back to the underlying circuit theory and semiconductor principles covered in the module.

# 5. Methodology

The thermistor is connected in series with a fixed 10 kΩ resistor between the 5V supply and ground, forming a voltage divider. The node between the thermistor and resistor is read by the Arduino's A0 analogue input. The Arduino converts this voltage into a resistance value, then applies the Beta parameter (Steinhart–Hart simplified) equation to estimate temperature in degrees Celsius. Two LEDs, each in series with a 220 ohm current-limiting resistor, are switched by Arduino digital output pins (D8 for blue/normal, D9 for red/warning) based on a defined threshold temperature. An I2C LCD module, wired to the Arduino's SDA (A4) and SCL (A5) pins, displays the live temperature reading and current status text. All components share a common ground rail on the breadboard, which itself connects back to a single Arduino GND pin.

# 6. Relevant Module Topics

This project draws on several study units from the ELYM 127 module study guide, listed below with how each applies to the build.

| Study unit | Topic | Application in this project |
| --- | --- | --- |
| Unit 1 | Circuit variables, Kirchhoff's laws, series resistors and voltage division | The thermistor + 10 kΩ resistor forms a textbook series voltage divider; the A0 reading is the divider's output voltage, calculated exactly the way voltage division is taught in this unit. |
| Unit 2 & 3 | General circuit analysis, Thévenin's theorem | The sensor branch can be reduced to a Thévenin equivalent when analysing how the Arduino's input impedance affects the reading — useful for explaining measurement error. |
| Unit 4 | Basic semiconductor theory, the junction diode | LEDs are junction diodes; understanding forward voltage drop and current limiting explains why the 220 ohm series resistor is required. |
| Unit 5 | Diodes and applications, optoelectronic devices | Directly covers LED operating principles, characteristic curves, and safe biasing — the theoretical basis for the blue/red indicator design. |
| Unit 6 | Bipolar junction transistors and applications | Relevant to a possible extension: driving a higher-current load (e.g. a fan or relay) from a digital pin would require a BJT switch, since the Arduino pin alone cannot supply enough current. |

# 7. Expected Outcomes

- A working prototype that accurately displays live temperature readings on the LCD.

- Reliable switching between the normal (blue) and warning (red) states at a defined threshold.

- A short written reflection connecting each hardware decision (resistor values, sensor choice, LED biasing) to the circuit theory in Units 1, 4, and 5.

# 8. Possible Extensions

- Add data logging (e.g. to an SD card) for temperature trends over time.

- Add an active buzzer for an audible warning alongside the visual one.

- Replace the fixed threshold with a potentiometer-adjustable one, introducing a second voltage divider.

# 9. Source Reference

Mawire, A. (Study guide compiler). ELYM 127 MEC — Introduction to Electronics. Department of Physics & Electronics, Faculty of Natural and Agricultural Sciences, North-West University.